Background/Nomenclature

Local and Global Coordinates and Orientations

(from global to local)

or equivalently:

where X/Y/Z are global coordinates, x/y/z are local coordinates,  is the DCM from global to local, and // are the unit vectors of the local coordinate system expressed in the global coordinate system.  The global coordinates are the inertia frame of the wind farm, with unit vectors , , and .  The local coordinates are such that x is along the rotor or wake-plane centerline (positive downwind), and y/z are transverse.  A cylindrical local coordinate system is also used where .

FAST.Farm Submodel Hierarchy

FAST.Farm Driver (PROGRAM FASTFarm)

Algorithm:

The transfer of the OpenFAST output to MoorDyn input (y_F to u_MD) involves point-to-point mesh mapping of motions.  The transfer of the MoorDyn output to OpenFAST input (y_MD to u_F) involves point-to-point mesh mapping of loads.

Input file checks, DEFAULTs, and unit conversions:

Simulation length (s):

Ambient wind model (-):

Ambient wind model (-):

Time step for low -resolution wind data input files; will be used as the global FAST.Farm time step (s):

Time step for high-resolution wind data input files (s):

Number  of low -resolution spatial nodes in each direction for wind data interpolation (-): , , and

Spacing of low -resolution spatial nodes in each direction for wind data interpolation (m): , , and

Number  of high-resolution spatial nodes in each direction for wind data interpolation (-): , , and

Number of wind turbines (-): ; note the turbine discretization is:

Spacing of high-resolution spatial nodes in each direction for wind data interpolation for each turbine (m): , , and

Radial increment of the wake plane finite-difference grid (m):

Number of radial nodes in the wake plane finite-difference grid: ; note the radial discretization is: ; the discretization of the equivalent Cartesian grid is  and

Number of wake planes: ; note the wake-plane discretization is:

Cutoff (corner) frequency of the low-pass time-filter for the wake advection, deflection, and meandering model (Hz):  (DEFAULT = 0.0007 Hz)

Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor (m):  (DEFAULT = 0.0)

Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor scaled with yaw error (m/deg):  (DEFAULT = 0.3); multiply by  when first read-in to convert from m/deg to m/rad

Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance (-):  (DEFAULT = 0.0)

Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance and yaw error (1/deg):  (DEFAULT = -0.004); multiply by  when first read-in to convert from 1/deg to 1/rad

Calibrated parameter for near-wake correction (-): (DEFAULT = 1.8)

Calibrated parameter for the influence of ambient turbulence in the eddy viscosity (-):  (DEFAULT = 0.05)

Calibrated parameter for the influence of the shear layer in the eddy viscosity (-):  (DEFAULT = 0.016)

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the minimum and exponential regions (-):  (DEFAULT = 0.0)

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the exponential and maximum regions (-):  (DEFAULT = 1.0)

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the value in the minimum region (-):  (DEFAULT = 1.0)

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the exponent in the exponential region (-):  (DEFAULT = 0.01)

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the minimum and exponential regions (-):  (DEFAULT = 3.0)

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the exponential and maximum regions (-):  (DEFAULT = 25.0)

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the functional value in the minimum region (-):  (DEFAULT = 0.2)

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the exponent in the exponential region (-):  (DEFAULT = 0.1)

Wake diameter calculation model (-):  (DEFAULT = 1)

Calibrated parameter for wake diameter calculation (-) [unused for ]:  (DEFAULT = 0.95)

Wake deficit model for wake deflection and meandering (-):  (DEFAULT = 1)

Spatial filter model for wake deflection and meandering (-):  (DEFAULT = 3)

Calibrated parameter for wake deflection and meandering (-):  (DEFAULT = 1.9)

Number of XY planes for output of disturbed wind data across the low-resolution domain (-):

Number of YZ planes for output of disturbed wind data across the low-resolution domain (-):

Number of XZ planes for output of disturbed wind data across the low-resolution domain (-):

Time step for disturbed wind visualization output (s) [unused for  and ]:  (DEFAULT = )

Number of radii for wake output for an individual rotor (-):

List of radial nodes for wake output for an individual rotor (-) [unused for ]:

Number of downstream distances for wake output for an individual rotor (-):

List of downstream distances for wake output for an individual rotor (m) [unused for ]:

Number of points for wind output (-):

Determine the number of time steps:

Calculate the number of low-resolution (FAST.Farm driver/glue code) time steps (-): ; note the time discretization is:

Calculate the number of high-resolution time steps per low-resolution time step:

Trigger a fatal error if the high-resolution time step is not an integer divisor of the low-resolution time step i.e. if

Calculate the number of low-resolution time steps per visualization output time step:

Trigger a fatal error if the visualization output time step is not an integer multiple of the low-resolution time step i.e. if

Warn the user if the number of wake planes is more than the number of low-resolution (FAST.Farm driver/glue code) time steps and reduce the number of wake planes accordingly (but still not less than 2):

Summary File Output (Written if SumPrint = TRUE)

FAST.Farm Summary File

Predictions were generated on DATE at TIME using FAST.Farm (VERSION, DATE), compiled as a XXX

compiled with

NWTC Subroutine Library (VERSION, DATE)

Super Controller (VERSION, DATE)

FAST Wrapper (VERSION, DATE) for OpenFAST (VERSION DATE)

MoorDyn (VERSION, DATE)

Wake Dynamics (VERSION, DATE)

Ambient Wind and Array Effects (VERSION, DATE)

Description from the FAST.Farm input file: DESCRIPTION

Ambient Wind:

Ambient wind model (-):  ()

Ambient wind input file/path:

Wind Turbines:

Turbine Number 	Output Turbine Number	X		Y		Z		OpenFAST Time Step	OpenFAST SubCycles		OpenFAST Input File

(-)					(-)							(m)	(m)	(m)	(s)							(-)								(-)

Wake Dynamics Finite-Difference Grid:  Radii ( Cartesian),  Planes

Radial Node Number		Output Node Number							Radius

(-)							(m)												(-)

Wake Dynamics Parameters

Cutoff (corner) frequency of the low-pass time-filter for the wake advection, deflection, and meandering model (Hz):  (low-pass time-filter parameter (-):)

Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor (m):

Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor scaled with yaw error (m/deg):

Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance (-):

Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance and yaw error (1/deg):

Calibrated parameter for near-wake correction (-):

Calibrated parameter for the influence of ambient turbulence in the eddy viscosity (-):

Calibrated parameter for the influence of the shear layer in the eddy viscosity (-):

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the minimum and exponential regions (-):

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the exponential and maximum regions (-):

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the value in the minimum region (-):

Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the exponent in the exponential region (-):

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the minimum and exponential regions (-):

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the exponential and maximum regions (-):

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the functional value in the minimum region (-):

Calibrated parameter in the eddy viscosity filter function for the shear layer defining the exponent in the exponential region (-):

Wake diameter calculation model (-):  ()

Calibrated parameter for wake diameter calculation (-):

Wake deficit model for wake deflection and meandering (-):  ()

Spatial filter model for wake deflection and meandering (-):  ()

Calibrated parameter for wake deflection and meandering (-):

Time Steps

Component							Time Step		Subcyles

(-)										(s)				(-)

FAST.Farm (glue code)							1

Super Controller									1

FAST Wrapper

Shared Mooring (MoorDyn)

Wake Dynamics									1

Ambient Wind and Array Effects				1

Low -resolution wind input						1

High-resolution wind input

Wind visualization output						^-1

FAST.Farm output files							1

Requested Channels in FAST.Farm Output Files: NUMBER

Number		Name		Units

0				Time		(s)

NUMBER		NAME	UNITS

Write Output

This is a list of all possible output parameters available within FAST.Farm (except those that are available from OpenFAST, which are specified within the FAST input file(s) and output separately for each turbine).  The names are grouped by meaning, but can be ordered in the OUTPUTS section of the FAST.Farm input file as you see fit.

Tα refers to turbine α, where α is a one-digit number in the range [1,9], corresponding to row α in wind turbine input table. If NumTurbines > 9, only values for the first 9 turbines can be output.  Setting α > NumTurbines yields invalid output.

Inβ and Otβ refer to super-controller input and output β, respectively, where β is a one-digit number in the range [1,9], corresponding to element β in the input and output arguments of the super-controller source code. If there are more than 9 elements, only values for the first 9 inputs and outputs can be output.  Setting β greater than the number of elements yields invalid output.

Nβ refers to radial output node β, where β is a two-digit number in the range [01,20], corresponding to entry β in the OutRadii list, where node β is at radius OutRadii[β]dr.  Setting β > NOutRadii yields invalid output.

Wβ refers to wind point β, where β is a one-digit number in the range [1,9], corresponding to entry β in the WindVelX, WindVelY, and WindVelZ lists.  Setting β > NWindVel yields invalid output.  Setting WindVelX, WindVelY, and WindVelZ outside the low-resolution wind domain also yields invalid output.

Dγ refers to downstream distance γ, where γ is a one-digit number in the range [1,9], corresponding to entry γ in the OutDist list.  Setting γ > NOutDist yields invalid output.  The output is also invalid if OutDist is a distance further downstream than the wake has been calculated or for any distance where the wake from the turbine has overlapped itself.

| Channel Name(s) | Unit(s) | Description |
| --- | --- | --- |
| Super Controller |  |  |
| SCGblInβ | (user) | Global (turbine-independent) super controller input β |
| SCTαInβ | (user) | Turbine-dependent super controller input β for turbine α |
| SCGblOtβ | (user) | Global (turbine-independent) super controller output β |
| SCTαOtβ | (user) | Turbine-dependent super controller output β for turbine α |
| Wind Turbine and its Inflow |  |  |
| RtAxsXTα, RtAxsYTα, RtAxsZTα | (-), (-), (-) | Orientation of the rotor centerline for turbine α in the global coordinate system |
| RtPosXTα, RtPosYTα, RtPosZTα | (m), (m), (m) | Position of the rotor (hub) center for turbine α in the global coordinate system |
| RtDiamTα | (m) | Rotor diameter for turbine α |
| YawErrTα | (deg) | Nacelle-yaw error for turbine α |
| TIAmbTα | (%) | Ambient turbulence intensity of the wind at the rotor disk for turbine α |
| RtVAmbTα | (m/s) | Rotor-disk-averaged ambient wind speed (normal to disk, not including structural motion, local induction or wakes from upstream turbines) for turbine α |
| RtVRelTα | (m/s) | Rotor-disk-averaged relative wind speed (normal to disk, including structural motion and wakes from upstream turbines, but not including local induction) for turbine α |
| CtTαNβ | (-) | Azimuthally averaged thrust force coefficient (normal to disk) for radial output node β of turbine α |
| Wake (for an Individual Rotor) |  |  |
| WkAxsXTαDγ, WkAxsYTαDγ, WkAxsZTαDγ | (-), (-), (-) | Orientation of the wake centerline for downstream distance γ of turbine α in the global coordinate system |
| WkPosXTαDγ, WkPosYTαDγ, WkPosZTαDγ | (m), (m), (m) | Center position of the wake centerline for downstream distance γ of turbine α in the global coordinate system |
| WkVelXTαDγ, WkVelYTαDγ, WkVelZTαDγ | (m/s), (m/s), (m/s) | Advection, deflection, and meandering velocity (not including the horizontal wake-deflection correction or low-pass time-filtering) of the wake for downstream distance γ of turbine α in the global coordinate system |
| WkDiamTαDγ | (m) | Wake diameter for downstream distance γ of turbine α |
| WkDfVxTαNβDγ, WkDfVrTαNβDγ | (m/s), (m/s) | Axial and radial wake velocity deficits for radial output node β and downstream distance γ of turbine α |
| EddVisTαNβDγ, EddAmbTαNβDγ, EddShrTαNβDγ | (m2/s), (m2/s), (m2/s) | Total eddy viscosity, and individual contributions to the eddy viscosity from ambient turbulence and the shear layer, for radial output node β and downstream distance γ of turbine α |
| Ambient Wind and Array Effects |  |  |
| WβVAmbX, WβVAmbY, WβVAmbZ | (m/s), (m/s), (m/s) | Ambient wind velocity (not including wakes) for point β in global coordinates (from the low-resolution domain) |
| WβVDisX, WβVDisY, WβVDisZ | (m/s), (m/s), (m/s) | Disturbed wind velocity (including wakes) for point β in the global coordinate system (from the low-resolution domain) |

These are calculated within the FAST.Farm driver as follows:

Super Controller

Wind Turbine and its Inflow

Wake (for an Individual Rotor)

In all of these calculations,  and  are related to distance  for turbine  as follows:

Ambient Wind and Array Effects

Super Controller (MODULE SuperController)

(Written in C)

Solves wind-farm super controller dynamics

| Inputs | Outputs | States (Discrete-Time) | Parameters |
| --- | --- | --- | --- |
| - – Global (turbine-independent) inputs (various units) - –Turbine-dependent inputs (various units) | - – Global (turbine-independent) outputs (various units) - –Turbine-dependent outputs (various units) | - – Global (turbine-independent) states (various units) - –Turbine-dependent states (various units) | - – Number of wind turbines (-) - – Time step (s) - – Global (turbine-independent) parameters (various units) - –Turbine-dependent parameters (various units) |

Initialization:

| Initialization Inputs | Initialization Outputs |
| --- | --- |
| - – Number of wind turbines (-) - – Time step (s) |  |

Set parameters from initialization inputs (,).

Set other parameters:

Initialize States:

Update States:

Note: Update states is written as a discrete-time system where the state update to  are based on inputs and states at  and parameters.

Calculate Outputs:

Note: Due to limitations in the driver/glue code, these calculations can only depend on the states at  and parameters; they cannot involve direct feedthrough of input to output. If the states are output directly, the routine does the following calculations.

FAST (MODULE FASTWrapper)

(One instance per turbine)

Solves aero-hydro-servo-elastic dynamics for an individual wind turbine

Note: Internal to OpenFAST there are many inputs, outputs, states, and parameters, what is listed here are the new variables needed in the wrapper for FAST.Farm.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Global (turbine-independent) commands from the super controller (various units) - –Turbine-dependent commands from the super controller (various units) - – Forces applied at various structural nodes of the floating platform (N) - – Moments applied at various structural nodes of the floating platform (Nm) - – UVW components of disturbed wind (ambient + wakes) across the high-resolution domain around the turbine for each high-resolution time step within a low-resolution time step (m/s) | - –Turbine-dependent commands to the super controller (various units) - – Displacements of various structural nodes of the floating platform (m) - – Orientations (DCMs) of various structural nodes of the floating platform (-) - – Orientation of rotor centerline, normal to disk (-) - – Nacelle-yaw error, i.e., the angle about positive from the rotor centerline to the rotor-disk-averaged relative wind velocity (ambient + wakes + turbine motion), both projected onto the horizontal plane (rad) - – Azimuth angle (from the nominally vertical axis in the disk plane, ) to the vector about which the inflow skew angle is defined, i.e., the angle about positive from to the vector about which the inflow skew angle is defined () (rad) - – Inflow skew angle () (rad) - – Center position of hub (m) - – Rotor diameter (m) - – Rotor-disk-averaged relative wind speed (ambient + wakes + turbine motion), normal to disk (m/s) - – Azimuthally averaged thrust force coefficient (normal to disk), distributed radially (-) - – Azimuthally averaged torque coefficient (normal to disk), distributed radially (-) |  | - – Number of radii in the radial finite-difference grid (-) - – Discretization of radial finite-difference grid (m) - – Number of high-resolution time steps per low-resolution time step (-) - – High-resolution time step (s) - – Number of high-resolution spatial nodes in X direction (-) - – Number of high-resolution spatial nodes in Y direction (-) - – Number of high-resolution spatial nodes in Z direction (-) - – X-component of the spatial discretization of the high-resolution spatial domain for this turbine (m) - – Y-component of the spatial discretization of the high-resolution spatial domain for this turbine (m) - – Z-component of the spatial discretization of the high-resolution spatial domain for this turbine (m) - – Number of OpenFAST time steps per low-resolution time step (-) - – Undisplaced global position of this turbine (m) |

Initialization:

| Initialization Inputs | Initialization Outputs |
| --- | --- |
| - – Number of radii in the radial finite-difference grid (-) - – Radial increment of radial finite-difference grid (m) - – Time step (s) - – Simulation length (s) - – Undisplaced global X-coordinate of this turbine (m) - – Undisplaced global Y-coordinate of this turbine (m) - – Undisplaced global Z-coordinate of this turbine (m) - – Filename of primary OpenFAST input file of this turbine (string) - – Number of high-resolution time steps per low-resolution time step (-) - – High-resolution time step (s) - – X-component of the origin of the high-resolution spatial domain for this turbine (m) - – Y-component of the origin of the high-resolution spatial domain for this turbine (m) - – Z-component of the origin of the high-resolution spatial domain for this turbine (m) - – Number of high-resolution spatial nodes in X direction (-) - – Number of high-resolution spatial nodes in Y direction (-) - – Number of high-resolution spatial nodes in Z direction (-) - – X-component of the spatial increment of the high-resolution spatial domain for this turbine (m) - – Y-component of the spatial increment of the high-resolution spatial domain for this turbine (m) - – Z-component of the spatial increment of the high-resolution spatial domain for this turbine (m) | - – OpenFAST time step (s) - – Gravity (m/s2) - – Water density (kg/m3) - – Water depth (m) - – Initial floating platform position and orientation (m, -) |

Set parameters from initialization inputs (,,,,,).

Set the radial discretization from the initialization inputs:

Set the discretization of the high-resolution spatial domain from the initialization inputs: , , and

Set the undisplaced position of this turbine in the global reference frame from the initialization inputs:

Note: This is the origin (0,0,0) of the inertial-frame coordinate system for this instance of OpenFAST.

CALL FAST_InitializeAll_T()

Trigger a fatal error if  doesn’t exist.

Calculate the number of OpenFAST time steps per low-resolution time step:

Trigger a fatal error if the OpenFAST time step is not an integer divisor of the low-resolution time step i.e. if

Trigger a fatal error if the simulation length is longer than that set for OpenFAST i.e. if

Set the reference positions and orientations of the input and output meshes:

Set the initialization outputs (, , , ), including:

t0

Calculates outputs at  based on inputs at

Note: This routine may have direct feedthrough of input to output

CALL FAST_Solution0_T()

Increment

Updates states and outputs to  based on inputs and states at  (this has an inherent time-step delay on outputs)

CALL FAST_Solution_T()

Note: In these calculations the commands from the super controller,  and , should be passed as inputs to the individual turbine controller.

Also, the forces and moments applied to the various structural nodes on the floating platform,  and , should be input to ElastoDyn or SubDyn, depending on CompSub, accordingly.

Also in these calculations the wind data should be interpolated within the 4D wind field stored in  based on ,, , , and .  Trigger a fatal error if any point () falls outside of this high-resolution spatial-temporal domain i.e. if , , , or

Calculate Outputs (Within t0 and Increment):

Turbine-dependent commands from the individual turbine controller to the super controller

Also, the displacements and orientations (DCMs) of the structural nodes on the floating platform,  and , should output from ElastoDyn or SubDyn, depending on CompSub, accordingly.

Calculate the outputs associated with the rotor disk and its inflow based on variables available within AeroDyn (following the nomenclature of AeroDynMesh.docx):

; Note:  within AeroDyn; Trigger a fatal error if the orientation of the rotor centerline or rotor-disk-averaged relative wind speed is directed vertically upward or downward () whereby the nacelle-yaw error is undefined i.e. if  or .

Derivation:

; Trigger a fatal error if the radius of the wake planes is not large relative to the rotor diameter i.e. if the diameter of the wake plane is not at least twice the rotor diameter i.e. if

To calculate outputs  and , we’ll use Line2-to-Line2 mesh-mapping to transfer the loads from deflected blade mesh of AeroDyn to a mesh that lies in the plane of the rotor disk, extending radially outward (without precone or blade sweep/curvature/deflection).  Because the new mesh does not deflect with the blade, we’ll need to perform a new mapping search and mapping transfer every time we want to calculate  and .  So, a reference mesh (with zero deflection) is stored for the new radial mesh based on the current hub position/orientation and the mesh associated with the deflected blade is saved as a temporary reference mesh (also with zero deflection).

; Note:

Calculate the aerodynamic blade loads on the new Line2-meshes (one for each blade) associated with calculating output :

Compute the azimuthally averaged thrust force and torque coefficients (normal to disk), distributed radially:

Derivations:

Wake Dynamics (MODULE WakeDynamics)

(One instance per rotor)

Calculates wake dynamics for an individual rotor, including:

Wake Advection, Deflection, and Meandering – Solves dynamic wake advection, deflection, and meandering for an individual rotor.

Near-Wake Correction – Calculates near-wake (pressure-gradient zone) correction to wake deficit for an individual rotor

Wake-Deficit Increment – Increments the quasi-steady axisymmetric wake deficit downwind for an individual rotor

Note: While the wake deficit formulation is implemented with discrete-time states, the calculations within a given wake plane propagation remain quasi-steady, as the theory necessitates. This is achieved by using discrete-time states to propagate parameters of the wake downstream with the wake planes.

For the start-up transient, whereby the wake has not yet propagated through all of the wake planes, the following limits are set to the number of wake planes to avoid having to set inputs, outputs, and states beyond where the wake has propagated:

For inputs,  is used in place of  for the values given at time  in both Update States and Calculate Output

For outputs,  is used in place of  for the values returned at time  in Calculate Output

For states,  is used in place of  for the values given at time  in both Update States Calculate Output and  is used in place of  for the values returned at time  in Update States

| Inputs | Outputs | States (Discrete-Time) | Parameters |
| --- | --- | --- | --- |
| - – Orientation of rotor centerline, normal to disk (-) - – Nacelle-yaw error, i.e., the angle about positive from the rotor centerline to the rotor-disk-averaged relative wind velocity (ambient + wakes + turbine motion), both projected onto the horizontal plane (rad) - – Azimuth angle (from the nominally vertical axis in the disk plane, ) to the vector about which the inflow skew angle is defined, i.e., the angle about positive from to the vector about which the inflow skew angle is defined () (rad) - – Inflow skew angle () (rad) - – Center position of hub (m) - – Advection, deflection, and meandering velocity of wake planes (m/s) - – Rotor-disk-averaged ambient wind speed, normal to disk (m/s) - – Ambient turbulence intensity of wind at rotor disk (-) - – Rotor diameter (m) - – Rotor-disk-averaged relative wind speed (ambient + wakes + turbine motion), normal to disk (m/s) - – Azimuthally averaged thrust force coefficient (normal to disk), distributed radially (-) - – Azimuthally averaged torque coefficient (normal to disk), distributed radially (-) | - – Orientations of wake planes, normal to wake planes (-) - – Center positions of wake planes (m) - – Axial wake velocity deficit at wake planes, distributed across the plane (m/s) - – Transverse horizontal wake velocity deficit at wake planes, distributed across the plane (m/s) - – Transverse nominally vertical wake velocity deficit at wake planes, distributed across the plane (m/s) - – Wake diameters at wake planes (m) | - – Orientations of wake planes, normal to wake planes (-) - – Time-filtered nacelle-yaw error at wake planes, i.e., the angle about positive from the rotor centerline to the rotor-disk-averaged relative wind velocity (ambient + wakes + turbine motion), both projected onto the horizontal plane (rad) - – Time-filtered azimuth angle (from the nominally vertical axis in the disk plane, ) to the vector about which the inflow skew angle is defined, i.e., the angle about positive from to the vector about which the inflow skew angle is defined () (rad) - – Time-filtered inflow skew angle () (rad) - – Time-filtered advection, deflection, and meandering velocity of wake planes (m/s) - – Center positions of wake planes (m) - – Downwind distance from rotor to each wake plane (m) - – Axial wake velocity deficit at wake planes, distributed radially (m/s) - – Radial wake velocity deficit at wake planes, distributed radially (m/s) - – Time-filtered rotor-disk-averaged ambient wind speed of wake planes, normal to planes (m/s) - – Time-filtered ambient turbulence intensity of wind at wake planes (-) - – Time-filtered rotor diameter associated with each wake plane (m) - – Time-filtered rotor-disk-averaged relative wind speed (ambient + wakes + turbine motion), normal to disk (m/s) - – Time-filtered azimuthally averaged thrust force coefficient (normal to disk), distributed radially (-) - – Time-filtered azimuthally averaged torque coefficient (normal to disk), distributed radially (-) | - – Number of wake planes downwind of the rotor where the wake is propagated (-) - – Number of radii in the radial finite-difference grid (-) - – Radial increment of radial finite-difference grid (m) - – Discretization of radial finite-difference grid (m) - – Horizontal discretization of each wake plane (m) - – Nominally vertical discretization of each wake plane (m) - – Time step (s) - – Low-pass time-filter parameter, with a value between 0 (minimum filtering) and 1 (maximum filtering) (exclusive) (-) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor (m) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor scaled with yaw error (m/rad) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance (-) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance and yaw error (1/rad) - – Calibrated parameter for near-wake correction (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the minimum and exponential regions (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the exponential and maximum regions (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the functional value in the minimum region (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the exponent in the exponential region (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the minimum and exponential regions (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the exponential and maximum regions (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the functional value in the minimum region (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the exponent in the exponential region (-) - – Calibrated parameter for the influence of ambient turbulence in the eddy viscosity (-) - – Calibrated parameter for the influence of the shear layer in the eddy viscosity (-) - – Wake diameter calculation model (-) - – Calibrated parameter for wake diameter calculation (-) |

Initialization:

| Initialization Inputs | Initialization Outputs |
| --- | --- |
| - – Number of wake planes downwind of the rotor where the wake is propagated (-) - – Number of radii in the radial finite-difference grid (-) - – Radial increment of radial finite-difference grid (m) - – Time step (s) - – Cutoff (corner) frequency of the low-pass time-filter for the wake advection, deflection, and meandering model (Hz) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor (m) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset at the rotor scaled with yaw error (m/rad) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance (-) - – Calibrated parameter in the correction for wake deflection defining the horizontal offset scaled with downstream distance and yaw error (1/rad) - – Calibrated parameter for near-wake correction (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the minimum and exponential regions (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the transitional diameter fraction between the exponential and maximum regions (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the functional value in the minimum region (-) - – Calibrated parameter in the eddy viscosity filter function for ambient turbulence defining the exponent in the exponential region (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the minimum and exponential regions (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the transitional diameter fraction between the exponential and maximum regions (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the functional value in the minimum region (-) - – Calibrated parameter in the eddy viscosity filter function for the shear layer defining the exponent in the exponential region (-) - – Calibrated parameter for the influence of ambient turbulence in the eddy viscosity (-) - – Calibrated parameter for the influence of the shear layer in the eddy viscosity (-) - – Wake diameter calculation model (-) - – Calibrated parameter for wake diameter calculation (-) |  |

Set parameters from initialization inputs (, , , , , , , , , , , , , , , , , , , , ).

Set the wake plane discretization from the initialization inputs: , , and

Set the low-pass time-filter parameter from the initialization input :

Initialize states:

The calculations in this section are for

Update States:

Wake Advection, Deflection, and Meandering

The calculations in this section are for

Apply the low-pass time-filter at the rotor to account for transients in inflow, turbine control and/or turbine motion on the wake.

Maintain the orientation of each wake plane as it propagates downstream.

Move the wake centers based on a passive tracer (in 3D).

Propagate the wake planes downstream: As a wake-plane propagates downstream, ensure that the ambient wind, ambient TI, and rotor diameter associated with the wake plane remain constant:

I wouldn’t expect this to ever happen (as the nacelle-yaw would have to rotate 180˚ almost instantaneously), but trigger a fatal error if the normalizing denominator in the wake-plane orientation time-filter is zero i.e. if

; Trigger a fatal error if the orientation of the rotor centerline at the rotor plane is directed vertically upward or downward () whereby the nacelle-yaw error and horizontal wake-deflection correction is undefined i.e. if .

; Trigger a fatal error if the time-filtered nacelle-yaw error at the rotor plane approaches  whereby the horizontal wake-deflection correction is invalid, i.e. if .

With the horizontal wake-deflection correction:

Derivation (for ):

Near-Wake Correction

Note: Even though a near-wake correction is applied, the wake deficit solution is still only accurate for downstream distances beyond about 3D. The near-wake correction is different between low thrust () and high thrust (). Trigger a fatal error if , as the wake model does not apply in the propeller brake region. Between these regions, a linear blending between the two solutions is implemented.  Where the disk-averaged thrust coefficient is (implemented via trapezoidal rule in the source code):

Low Thrust ():

Derive the local axial induction factor from the time-filtered local thrust coefficient, based on the momentum region without tip loss; the local thrust coefficient is clipped at .

The analytical expression for the wake-deficit boundary condition at the disk is:

Derivation:  results from mass conservation within an annulus: , which leads to: , which leads to: , where  is in the rotor disk where the induced velocity is  and  is in a wake plane downstream where the induced velocity has dropped to

Numerical implementation:

with the local downstream radius solved via the trapezoidal rule:

High Thrust ():

This is a Gaussian fit to LES solutions at high thrust/

Where:

Wake-Deficit Increment

Assuming the thin-shear-layer approximation to Navier Stokes (no pressure term, axisymmetric, velocity gradients much bigger in radial direction than axial direction) and steady state, the analytical expressions for the conservation of momentum and conservation of mass (continuity) are:

(momentum)

(continuity)

with the eddy viscosity:

where the eddy viscosity filter functions, representing the delay in the turbulent stress generated by ambient turbulence and the development of turbulent stresses generated by the shear layer are:

and the characteristic length scale is:

The remaining calculations below are in a loop for

Calculate the downwind increment of wake planes:  (which equals  from above)

Calculate the characteristic length scale used in the eddy viscosity formulation, the radial gradient of the axial wake deficit used in the eddy viscosity formulation (using a central finite-difference scheme), the eddy viscosity, and the radial gradient of the eddy viscosity (using a central finite-difference scheme):

Note: The equations at  are based on the fact that  and , uniquely for each

If  for a given wake plane (), then the wake deficit is simply propagated to the next wake plane at the next time step:

Otherwise (), solve the terms of the numerical momentum and continuity equations.

The numerical implementations are based on a second-order accurate finite-difference scheme at , following the implicit Crank-Nicolson method. That is, central differences are used for all derivatives. A different finite-difference scheme is needed for the momentum and continuity equations because when the same finite-difference scheme used for momentum is used for continuity, the resulting tridiagonal matrix is not diagonally dominant. For the momentum equation, the terms , , , and  are calculated at  (or equivalently, ) to avoid nonlinearities in the solution for  (this will lead to error that will prevent the solution from achieving second-order convergence). For the continuity equation, the finite-difference scheme is based on a second-order accurate finite-difference scheme at  and , except that  and  are computed at  (or equivalently, ) to avoid numerical instabilities.

Momentum:

where

Continuity:

Derivations: Define the terms of the numerical momentum and continuity equations for  based on a second-order accurate finite-difference scheme at , following the implicit Crank-Nicolson method. That is, central differences are used for all derivatives. A different finite-difference scheme is needed for the momentum and continuity equations because when the same finite-difference scheme used for momentum is used for continuity, the resulting tridiagonal matrix is not diagonally dominant. For the momentum equation, the terms , , , and  are calculated at  (or equivalently, ) to avoid nonlinearities in the solution for  (this will lead to error that will prevent the solution from achieving second-order convergence). For the continuity equation, the finite-difference scheme is based on a second-order accurate finite-difference scheme at  and , except that  and  are computed at  (or equivalently, ) to avoid numerical instabilities.

Momentum:

with

Note: The equations at  are based on the fact that

Therefore:

where

Continuity:

with

After looping through , we need to calculate the eddy viscosity on the most downwind wake plane () for write output purposes:

Calculate Output

The calculations in this section are for

Return the current values of the discrete-time states:

Note: This output is one time-step delayed, but due to slow wake dynamics, this is OK to avoid having direct feedthrough of input to output and thus simplifying the glue code.

Private SUBROUTINEs / FUNCTIONs

Azimuth Average (FUNCTION AzimAvg)

Converts a scalar array from a Cartesian system in Cartesian coordinates to an axisymmetric system in radial coordinates by azimuth averaging:

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Scalar field distributed across the Cartesian plane (-) - – Horizontal discretization of Cartesian grid (m) - – Nominally vertical discretization of Cartesian grid (m) - – Discretization of radial finite-difference grid (m) | - – Azimuth-averaged scalar field, distributed radially |  |  |

Axisymmetric to Cartesian (SUBROUTINE Axisymmetric2Cartesian)

Converts velocity vectors from an axisymmetric system in radial coordinates to a Cartesian system in Cartesian coordinates:

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Axial velocity, distributed radially (m/s) - – Radial velocity deficit, distributed radially (m/s) - – Discretization of radial finite-difference grid (m) - – Horizontal discretization of Cartesian grid (m) - – Nominally vertical discretization of Cartesian grid (m) | - – Axial velocity, distributed across the plane (m/s) - – Transverse horizontal velocity, distributed across the plane (m/s) - – Transverse nominally vertical velocity, distributed across the plane (m/s) |  |  |

Eddy Viscosity Filter Function (FUNCTION EddyFilter)

Calculates the eddy viscosity filter functions, representing the delay in the turbulent stress generated by ambient turbulence or the development of turbulent stresses generated by the shear layer:

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Downwind distance from rotor to wake plane (m) - – Rotor diameter associated with wake plane (m) - – Calibrated parameter in the eddy viscosity filter function defining the transitional diameter fraction between the minimum and exponential regions (-) - – Calibrated parameter in the eddy viscosity filter function defining the transitional diameter fraction between the exponential and maximum regions (-) - – Calibrated parameter in the eddy viscosity filter function defining the functional value in the minimum region (-) - – Calibrated parameter in the eddy viscosity filter function defining the exponent in the exponential region (-) | - – Filter function (-) |  |  |

Trigger a fatal error if the inputs are invalid:

Trigger a fatal error if the calibrated parameter is outside of a valid range i.e. if , , , or

Wake Diameter (FUNCTION WakeDiam)

Calculates the wake diameter at a wake plane, based on one of four models:

Model 1: The wake diameter is taken to be the rotor diameter,

Model 2: The wake diameter is taken to be the largest of the rotor diameter or the diameter at which the axial velocity of the wake is the  fraction of the ambient wind speed.

The analytical expression is:

Model 3: The wake diameter is taken to be the largest of the rotor diameter or the diameter that captures the  fraction of the mass flux of the axial wake deficit across the wake plane.

The analytical expression is:

Model 4: The wake diameter is taken to be the largest of the rotor diameter or the diameter that captures the  fraction of the momentum flux of the axial wake deficit across the wake plane.

The analytical expression is:

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Axial wake velocity deficit at wake plane, distributed radially (m/s) - – Rotor-disk-averaged ambient wind speed of wake plane, normal to plane (m/s) - – Rotor diameter associated with wake plane (m) - – Radial increment of radial finite-difference grid - – Wake diameter calculation model (-) - – Calibrated parameter for wake diameter calculation (-) | - – Wake diameter (m) |  |  |

: Model 1

: Model 2

Trigger a fatal error if the calibrated parameter is outside of a valid range i.e. if

Numerical implementation:

where  is used to ensure that the wake diameter is at least as large as the rotor diameter and in case there is more than one value of  that satisfies the criteria.

: Model 3

Trigger a fatal error if the radial increment is zero i.e. if

Trigger a fatal error if the calibrated parameter is outside of a valid range i.e. if

Calculate the number of radii in the radial finite-difference grid:

Derive the radial discretization from the initialization inputs:

Calculate the radial-dependent mass-flux integral via the trapezoidal rule:

Numerical implementation:

where  is used to ensure that the wake diameter is at least as large as the rotor diameter and in case there is more than one value of  that satisfies the criteria.

: Model 4

Trigger a fatal error if the radial increment is zero i.e. if

Trigger a fatal error if the calibrated parameter is outside of a valid range i.e. if

Calculate the number of radii in the radial finite-difference grid:

Derive the radial discretization from the initialization inputs:

Calculate the radial-dependent momentum-flux integral via the trapezoidal rule:

Numerical implementation:

where  is used to ensure that the wake diameter is at least as large as the rotor diameter and in case there is more than one value of  that satisfies the criteria.

Trigger a fatal error about an invalid

Thomas Algorithm (FUNCTION ThomasAlgorithm)

Solves the tridiagonal linear system for  using the Thomas algorithm:

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Sub diagonal (-) - – Main diagonal (-) - – Super diagonal (-) - – Right-hand side (-) | - – Solution of the linear solve (-) |  |  |

Calculate the size of the matrix:

Note: all of these vectors should be the same size;  and  should be the same size or one element smaller (if the same size, the one remaining element each,  and , are unused).  Trigger a fatal error if the vectors are not the right size.

Trigger a fatal error if the tridiagonal matrix is not diagonally dominant i.e. if:

The Thomas algorithm is (overwriting the original arrays):

Step forward to eliminate  from the tridiagonal matrix:

Step backward to solve for :

Ambient Wind and Array Effects (MODULE Wind_AmbientAndArray)

Processes ambient wind and wake interactions across the wind farm, including:

Ambient Wind – Process ambient wind across the wind farm from either a high-fidelity precursor simulation or an interface to the OpenFAST InflowWind module

Wake Merging – Identifies zones of overlap between all wakes across the wind farm and merges their wake deficits

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Orientations of wake planes, normal to wake planes, for each turbine (-) - – Center positions of wake planes for each turbine (m) - – Axial wake velocity deficit at wake planes, distributed across the plane, for each turbine (m/s) - – Transverse horizontal wake velocity deficit at wake planes, distributed across the plane, for each turbine (m/s) - – Transverse nominally vertical wake velocity deficit at wake planes, distributed across the plane, for each turbine (m/s) - – Wake diameters at wake planes for each turbine (m) | - – UVW components of disturbed wind (ambient + wakes) across each high-resolution domain around a turbine for each high-resolution time step within a low-resolution time step (m/s) - – Advection, deflection, and meandering velocity of wake planes for each turbine (m/s) - – Rotor-disk-averaged ambient wind speed, normal to disk, for each turbine (m/s) - – Ambient turbulence intensity of wind at rotor disk for each turbine (-) |  | - – Ambient wind model (-) - – Low-resolution (FAST.Farm driver/glue code) time step (s) - – High-resolution time step (s) - – Number of low-resolution (FAST.Farm driver time steps (-) - – Number of wind turbines (-) - – Number of wake planes downwind of the rotor where the wake is propagated (-) - – Number of radii in the radial finite-difference grid (-) - – Horizontal discretization of each wake plane (m) - – Nominally vertical discretization of each wake plane (m) - – Number of low-resolution spatial nodes in X direction (-) - – Number of low-resolution spatial nodes in Y direction (-) - – Number of low-resolution spatial nodes in Z direction (-) - – XYZ-components of the spatial increment of the low-resolution domain (m) - – XYZ components (global positions) of the spatial discretization of the low-resolution spatial domain (m) - – Number of high-resolution time steps per low-resolution time step (-) - – Number of high-resolution spatial nodes in X direction (-) - – Number of high-resolution spatial nodes in Y direction (-) - – Number of high-resolution spatial nodes in Z direction (-) - – XYZ components (global positions) of the spatial discretization of the high-resolution spatial domain for each turbine (m) - – Spatial resolution of the polar grid for each wake plane of each turbine (m) - – XYZ-components (global positions) of each undisplaced turbine (m) - – Maximum possible number of points in the polar grid for the wake plane at each rotor - – Wake deficit model for wake meandering (-) - – Spatial filter model for wake meandering (-) - – Calibrated parameter for wake meandering (-) - – Normalized wake plane radius for wake meandering (normalized by the wake diameter) (-) - – Path name of wind data files from high-fidelity precursor (string) - – Root name (including path) of the FAST.Farm primary input file (string) - – Number of low-resolution time steps per visualization output time step (-) - – Flag for writing full visualization output (flag) - – Number of XY planes for visualization output of disturbed wind data (-) - – Z coordinates of XY planes for visualization output of disturbed wind data (m) - – Number of YZ planes for visualization output of disturbed wind data (-) - – X coordinates of YZ planes for visualization output of disturbed wind data (m) - – Number of XZ planes for visualization output of disturbed wind data (-) - – Y coordinates of XZ planes for visualization output of disturbed wind data (m) |

Initialization:

| Initialization Inputs | Initialization Outputs |
| --- | --- |
| - – Low-resolution (FAST.Farm driver/glue code) time step (s) - – High-resolution time step (s) - – Number of low-resolution (FAST.Farm driver time steps (-) - – Number of high-resolution time steps per low-resolution time step (-) - – Number of wind turbines (-) - – Number of wake planes downwind of the rotor where the wake is propagated (-) - – Number of radii in the radial finite-difference grid (-) - – Radial increment of radial finite-difference grid (m) - – Wake deficit model for wake meandering (-) - – Spatial filter model for wake meandering (-) - – Calibrated parameter for wake meandering (-) - – Ambient wind model (-) - – Path name of wind data files from high-fidelity precursor (string) - – Number of low-resolution spatial nodes in X direction (-) - – Number of low-resolution spatial nodes in Y direction (-) - – Number of low-resolution spatial nodes in Z direction (-) - – X-component of the origin of the low-resolution spatial domain (m) - – Y-component of the origin of the low-resolution spatial domain (m) - – Z-component of the origin of the low-resolution spatial domain (m) - – X-component of the spatial increment of the low-resolution spatial domain (m) - – Y-component of the spatial increment of the low-resolution spatial domain (m) - – Z-component of the spatial increment of the low-resolution spatial domain (m) - – Number of high-resolution spatial nodes in X direction (-) - – Number of high-resolution spatial nodes in Y direction (-) - – Number of high-resolution spatial nodes in Z direction (-) - – X-component of the origin of the high-resolution spatial domain for each turbine (m) - – Y-component of the origin of the high-resolution spatial domain for each turbine (m) - – Z-component of the origin of the high-resolution spatial domain for each turbine (m) - – X-component of the spatial increment of the high-resolution spatial domain for each turbine (m) - – Y-component of the spatial increment of the high-resolution spatial domain for each turbine (m) - – Z-component of the spatial increment of the high-resolution spatial domain for each turbine (m) - – Undisplaced global X-coordinate of each turbine (m) - – Undisplaced global Y-coordinate of each turbine (m) - – Undisplaced global Z-coordinate of each turbine (m) - – Root name (including path) of the FAST.Farm primary input file (string) - – Number of low-resolution time steps per visualization output time step (-) - – Flag for writing full visualization output (flag) - – Number of XY planes for visualization output of disturbed wind data (-) - – Z coordinates of XY planes for visualization output of disturbed wind data (m) - – Number of YZ planes for visualization output of disturbed wind data (-) - – X coordinates of YZ planes for visualization output of disturbed wind data (m) - – Number of XZ planes for visualization output of disturbed wind data (-) - – Y coordinates of XZ planes for visualization output of disturbed wind data (m) | - – Number of high-resolution spatial nodes in X direction (-) - – Number of high-resolution spatial nodes in Y direction (-) - – Number of high-resolution spatial nodes in Z direction (-) - – X-component of the origin of the high-resolution spatial domain for each turbine (m) - – Y-component of the origin of the high-resolution spatial domain for each turbine (m) - – Z-component of the origin of the high-resolution spatial domain for each turbine (m) - – X-component of the spatial increment of the high-resolution spatial domain for each turbine (m) - – Y-component of the spatial increment of the high-resolution spatial domain for each turbine (m) - – Z-component of the spatial increment of the high-resolution spatial domain for each turbine (m) |

Set parameters from initialization inputs (, , , , , , , , , , , , , , , , , , , , ).

Set the wake plane discretization from the initialization inputs:  and

Set the undisplaced position of each turbine:

Ensure that enough wind data has been generated:

Trigger a fatal error if there is not enough low-resolution wind data i.e. if  does not exist (if there are more low-resolution wind data files, we’ll only use the first )

Trigger a fatal error if there are not at least as many high-resolution wind data files as there are wind turbines and high-resolution time steps i.e. if

does not exist (if there are more high-resolution wind data files, we’ll only use the first  and )

Read-in low- and high-resolution spatial discretization data:

Read-in , , , , , , , , and  from

Read-in , , , , , , , , and  from

Trigger a fatal error if there are not at least two low-resolution spatial nodes in each direction i.e. if , , or ; note the low-resolution discretization is: , , , and

Trigger a fatal error if the number of high-resolution spatial nodes in each direction is not identical between files i.e. if , , or  and not at least two i.e. if , , or ; note the high-resolution discretization is: , , , and

Trigger a fatal error if the low-resolution spacing in each direction is not positive i.e. if , , or

Trigger a fatal error if the high-resolution spacing for each turbine in each direction is not positive i.e. if , , or

CALL InflowWind_Init

CALL InflowWind_Init

CALL InflowWind_Init

Trigger a fatal error about an invalid

Set the discretization of the low-resolution spatial domain from the initialization inputs:

Set the discretization of each high-resolution spatial domain from the initialization inputs:

Set the normalized wake-plane radius for wake meandering (normalized by the wake diameter):

Set the spatial discretization of the polar grid for each wake plane:

Estimate the maximum number of points in the polar grid for the wake plane at each rotor:

CALL AWAE_US at  to read-in the ambient wind for the initial calculate output.

Update States:

IF  THEN

Read-in ambient wind data from low- and high-resolution domains:

Read-in  from

Read-in  from

ELSE

END IF

Calculate Output:

The calculations in this section make use of wake volumes, which are volumes formed by a (possibly curved square cylinder starting at the wake plane and ending axially downstream until it is sliced by the next wake plane.

Local variables:

- – UVW components of ambient wind across the low-resolution domain throughout the farm (m/s)

- – UVW components of disturbed wind (ambient + wakes) across the low-resolution domain throughout the farm (m/s)

- – 3x3 matrix at each point in the low-resolution domain that scales the contribution of the wake deficit in the disturbed wind field (-)

- – UVW components of ambient wind across each high-resolution domain around a turbine (one for each turbine) for each high-resolution time step within a low-resolution time step (m/s)

- – Cosine of the angle between the starting and ending wake planes of a given wake volume (-)

- – Sine of the angle between the starting and ending wake planes of a given wake volume (-)

- – Logical flag indicating whether or not the starting and ending wake planes of a given wake volume are parallel (flag)

- – Radius to the wake center in the starting wake plane from the line where the starting and ending wake planes intersect for a given wake volume (-)

- – Radius to the wake center in the ending wake plane from the line where the starting and ending wake planes intersect for a given wake volume (-)

- – Radial orientation to the wake center in the starting wake plane from the line where the starting and ending wake planes intersect for a given wake volume (-)

- – Radial orientation to the wake center in the ending wake plane from the line where the starting and ending wake planes intersect for a given wake volume (-)

- – Position of the point on the line where the starting and ending wake planes intersect for a given wake volume where the wake center of the starting wake plane projects normal to the line (-)

- – Position of the point on the line where the starting and ending wake planes intersect for a given wake volume where the wake center of the end wake plane projects normal to the line (-)

- – Horizontal orientations (horizontal to the XY plane), normal to wake centerlines/, for each wake plane of each turbine (-)

- – Nominally vertical orientations, normal to both wake centerlines/ and the horizontal/, for each wake plane of each turbine (tilt will cause  to be slanted from the actual vertical) (-)

- – Number of wakes volumes overlapping a given point in the wind spatial domain (-); note the wake discretization is:  and

- – Indices of the wake planes associated with a given point in the wind spatial domain (-)

- – Indices of the turbines associated with a given point in the wind spatial domain (-)

- – Orientations of wake planes, normal to wake planes, associated with a given point in the wind spatial domain (-)

- – Weighted-average orientation of wake plane, normal to wake plane, associated with a given point in the wind spatial domain (weighted by the magnitude of each axial wake deficit) (-)

- – Distance to a given point in the wind spatial domain from the starting wake plane

- – Distance to a given point in the wind spatial domain from the ending wake plane

- – Fractional distance between wake planes, associated with a given point in the wind spatial domain, where

- – Center position of the wake plane associated with a given point in the wind spatial domain (m)

- – Horizontal distances to the wake centerlines, associated with a given point in the wind spatial domain (m)

- – Nominally vertical distances to the wake centerlines, associated with a given point in the wind spatial domain (m)

- – Horizontal orientations (horizontal to the XY plane), normal to wake centerlines/, associated with a given point in the wind spatial domain (-)

- – Nominally vertical orientations, normal to both wake centerlines/ and the horizontal/, associated with a given point in the wind spatial domain (tilt will cause  to be slanted from the actual vertical) (-)

- – Axial wake velocity deficits at wake planes, associated with a given point in the wind spatial domain (m/s)

- – Transverse horizontal wake velocity deficits at wake planes, associated with a given point in the wind spatial domain (m/s)

- – Transverse nominally vertical wake velocity deficits at wake planes, associated with a given point in the wind spatial domain (m/s)

- – Number of points in the polar grid for each wake plane of each turbine (-); note the polar grid discretization is:

- – Number of radial points in the polar grid for each wake plane of each turbine (-); note the radial discretization is:

- – Number of azimuthal points in the polar grid for each wake plane of each turbine (-); note the azimuthal discretization is:

- – Radial distances from the wake centerlines to points in the polar grid for each wake plane of each turbine (m)

- – Azimuth angles about  from horizontal/ to points in the polar grid for each wake plane of each turbine (rad)

- – Positions to points in the polar grid for each wake plane of each turbine (m)

- – UVW components of ambient wind at points in the polar grid for each wake plane for each turbine (m/s)

- – UVW components of disturbed wind (ambient + wakes) at points in the polar grid for each wake plane for each turbine (m/s)

- – Weighting for spatial averaging/filtering for points in the polar grid for each wake plane of each turbine (m)

- – Logical flag indicating whether or not the points in the polar grid for each wake plane of each turbine lie within the low-resolution domain (flag)

- – Spatial-averaged ambient wind speed in the low-resolution domain associated with the wake plane at the rotor disk, for each turbine (m/s)

Ambient Wind

The analytical expressions for the rotor-disk-averaged ambient wind speed, normal to the disk, for each turbine, , and the ambient turbulence intensity for each turbine, , are based on the wake plane at the rotor disk, , associated with that turbine and based on a diameter equal to  times the wake diameter, . The ambient turbulence intensity is based on a spatial- (instead of time-) average of the three vector components (instead of just the axial component). In the spatial averaging, a uniform weighting is always applied:

Where:

Wake Merging

The disturbed (ambient + wakes) wind at a given point in the wind spatial domain for both the low- and high-resolution domains,  and , is found by combining the local ambient wind with a Root-Sum-Squared (RSS) of the axial velocity deficits and a straight vector sum of the transverse velocity deficits for all wake volumes overlapping with the given point in the wind spatial domain based on a square cylinder width/height equal to the width/height of the wake planes, . Because each wake plane may have a unique orientation, what constitutes “axial” and “transverse” at a given point in the wind spatial domain is determined by weighted-averaging the orientation of each wake plane overlapping that point (weighted by the magnitude of each axial wake deficit). For the high-resolution domains (needed to calculate the disturbed wind-inflow to a turbine), “for…otherwise” statements are used in the summations to prevent a turbine from interacting with its own wake. For the low-resolution domain, the wake deficit contribution can be kept in full (),accounted for only in the along-wind direction (), or ignored altogether ().

The advection, deflection, and meandering velocity of wake planes for each turbine, , is found by spatially averaging the disturbed (ambient + wakes) wind velocities for a polar grid of points in the plane based on a diameter equal to  times the wake diameter, .  In the spatial averaging, the disturbed wind velocity of each point is weighted based on its radius from the wake center, depending on the spatial filter model.

The analytical expressions are as follows:

where:

Numerical implementation (for both Ambient Wind and Wake Merging):

Warnings and errors that can be triggered:

Issue a warning if the center of any wake plane for any turbine has left the low-resolution domain i.e. if ., , , , , or .

I wouldn’t expect this to every happen, but trigger a fatal error if the magnitude of the spatial-averaged ambient wind speed in the low-resolution domain associated with the wake plane at the rotor disk, for a turbine is zero such that the ambient turbulence intensity is undefined i.e. if .

First set local variables that are constant for a given wake volume at this time step:

For the low-resolution domain:

For the high-resolution domains:

Visualization Output

If , write the disturbed wind from the low- and high-resolution domains every visualization output time step:

Write  to

Write  to

If requested and , write planar slices of disturbed wind through the low-resolution domain every visualization output time step:

Private SUBROUTINES

jinc (SUBROUTINE jinc)

Calculates the jinc function by calling the Bessel function of the first kind:

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – normalized radius (-) | - – jinc function (-) |  |  |

INTERP3D (SUBROUTINE INTERP3D)

Interpolates the 3D velocity field  at input position  within the uniform rectangular grid defined by  and  using a trilinear isoparametric interpolation.  If  does not lie within the boundaries of the uniform rectangular grid, flag  is set to .FALSE.

e.g.

| Inputs | Outputs | States | Parameters |
| --- | --- | --- | --- |
| - – Position where the 3D velocity field will be interpolated (m) - – 3D velocity field to be interpolated (m/s) - – Origin of the spatial domain (m) - – XYZ-components of the spatial increment of the domain (m) | - – Interpolated velocity (m/s) - – Logical flag indicating whether or not the input position lies within the domain (flag) - Option: – Velocities at the 8 points in the domain surrounding (m/s) |  |  |

Check bounds:

Interpolate:

The ambient turbulence intensity is based on a spatial- (instead of time-) average of the three vector components (instead of just the axial component).