# Using REDWIN DLLs with OpenFAST (and other codes)

Sample code is distributed with the REDWIN DLLs that illustrates two methods of linking the DLL into a code.  The first method is a static linking at compile time, and the second method is dynamically loading the DLL at run time.  When coupling the REDWIN DLL into other codes, such as OpenFAST, we recommend the dynamic linking method outlined in section A2, ‘DLL not compiled together with simulation software’ of Appendix A of the documentation for the DLL (20150014-11-R_Rev0_3D_Foundation Model Library.pdf).  This method allows for loading different model versions of the DLL as needed.  This document is intended as a companion to the REDWIN documentation with additional lessons learned through incorporating the DLLs into the SoilDyn module in OpenFAST.

The SoilDyn module with example test cases and a driver will be released in an update to OpenFAST. This may provide additional hints not covered in this document to developers looking to incorporate REDWIN into their codes.

## General tips

DLL linking:

- The code provided with the REDWIN DLLs includes an example for dynamically linking (testModel1_v2.f90).  This illustrates how to load and call the DLL.

USE kernel32, ONLY    : FreeLibrary
INTEGER(BOOL)        :: Success

Success = FreeLibrary( FileAddr )	! True if success. FileAddr is address

! from call to load library call

To avoid potential memory leaks, unload the library with the ‘FreeLibrary’ function when ending the simulation (this is not given in the sample code).

! Kind for eight-byte floating-point numbers

INTEGER, PARAMETER :: R8Ki = SELECTED_REAL_KIND( 14, 300 )

TYPE, PUBLIC :: REDWINdllType

character(45)  :: PROPSFILE      !< Name of Properties file

character(45)  :: LDISPFILE      !< Load-displacement curve file

INTEGER        :: IDtask         !< Task identifier for DLL

INTEGER        :: nErrorCode     !< number of returned error codes

INTEGER,    DIMENSION(1:100)      :: ErrorCode  !< Array of error codes

REAL(R8Ki), DIMENSION(1:100,1:200):: Props      !< model properties

REAL(R8Ki), DIMENSION(1:12,1:100) :: StVar      !< state variables

INTEGER,    DIMENSION(1:12,1:100) :: StVarPrint !< Unused placeholder

REAL(R8Ki), DIMENSION(1:6)        :: Disp       !< Input displacements

REAL(R8Ki), DIMENSION(1:6)        :: Force      !< Returned forces

REAL(R8Ki), DIMENSION(1:6,1:6)    :: D          !< 6x6 stiffness matrix

END TYPE REDWINdllType

TYPE(REDWINdllType) :: DLLdata(4)	  !Array of 4 REDWINdllType derived types

It is possible to use a single DLL for soil interactions at multiple different structural points simultaneously.  This requires keeping separate copies of all arguments passed to the DLL for each soil interaction point.  In Fortran, this can be simplified with the use of an array of a derived type as follows:

- When dynamic linking of the DLL, the DLL may be located at any convenient location; it does not need to be located with the simulation input data or the executable.

Arguments for DLL:

See Table 1 in 20150014-11-R_Rev0_3D_Foundation Model Library.pdf for detailed descriptions of arguments passed to the DLL.

- Locate the PROPSFILE and LDISPFILE used by the DLL in the current working directory. If the files are not found, a segmentation fault may occur.  We recommend checking the existence of these files prior to calling the DLL.

- The StVar array contains the state information for a given soil interaction point.  It may be possible to reset the interaction to a prior state by resetting this to a previous set of values, e.g., for correction steps in time integration.

- The error codes returned in the ErrorCode array are specific to each DLL model.  At present, there is no simple method of determining which DLL model has been loaded: none of the returned values appear to indicate this. This presents challenges in parsing the returned errors in the calling code.

Other notes:

- The coordinate system used within the REDWIN DLLs does not match the standard wind turbine (IEC) convention used by OpenFAST.  However, no coordinate transforms have been required as the Y and Z axis only have sign changes and the REDWIN DLL does not exhibit direction preference in displacement.

- If the REDWIN DLL encounters an error, it will abort the program without returning any error codes.