FAST.Farm Wake Extent and Buffering

FAST.Farm Input File Changes

Distance of full wake propagation, expressed as a multiple of , beyond which the wake enters the buffer region;  (DEFAULT = 15)

Length of wake propagation buffer region, expressed as a multiple of , beyond which wake is not tracked; the wake deficit is linearly scaled to zero from the end of full wake region to the end of the buffer region;  (DEFAULT = 5)

These new inputs are used to set new parameters in Wake Dynamics (see the calculations below):

Propagation distance defining region of full wake

Propagation distance defining region of full wake plus buffer wake

() is removed as an input to FAST.Farm, but will be calculated dynamically at run time for each turbine at each time step based on the new inputs (see below).

Changes to Wake Dynamics Calculations:

is changed from a parameter of FAST.Farm to a discrete-time state, initialized in WD_Init(), computed within WD_US(), and output from WD_CO() (that is, the current state is output) and input to AWAE_CO() for use there.  In WD_US() at each time step, a single new plane is added to the set of wake planes at the rotor and other planes are propagated downstream.  At initialization,  is initialized to 2 to ensure that there is at least one wake volume needed by AWAE.  The loops through wake planes in FAST.Farm that are currently time-step dependent (e.g.,  or ) in WD and AWAE most now based on state/input/output  (i.e., ).  FAST.Farm should abort if  ever gets less than 2 (e.g., if the wake extent is very small), because in that case there are no wake volumes needed by AWAE.  FAST.Farm should also abort if wake planes pass each other.  Rather than reallocating the size of the states, inputs, and outputs of FAST.Farm currently sized to  to be dynamically allocated to size , a maximum number of wake planes is estimated, , and the arrays are allocated to that.  Any data stored in the states, inputs, and outputs for wake planes beyond  are ignored in the calculations.

In the buffer region, the wake deficits are linearly scaled from full down to zero.  That is, in WD_CalcOutput(), after calculating outputs , , and  at wake plane , scale down their values in the buffer region.

In WD_Init():

Derivation for  = 1

And for  = 2,3

Initialize the state tracking the number of wake planes

In WD_US(), after updating  to the next time step:

In WD_CO():

(i.e., the output  equals the current value of its state)

linearly scale-down the wake deficit in the buffer region